Vienna Building Model Parameters Appendix

Generated at UTC: 2026-04-16T15:34:37+00:00

Scope

• Appendix scope: Vienna cohort-based building model parameters for the current paper path.

• Active runtime variant: thermal_archetypes.variant = calibrated_v1.

• The appendix separates literature-backed fields, pragmatic V1 assumptions, and legacy compatibility fields.

• Case-specific paper JSON overrides can change the global control and thermflex settings; the archetype and calibrated sidecar tables here are the repo SSOT at generation time.

Notes

• The calibrated_v1 layer keeps the base envelope and geometry fields intact and adds calibration_v1 as a sidecar payload.

• Runtime thermal dynamics use the calibrated reduced-order fields (effective loss coefficient, effective heat capacity, and tau) when calibrated_v1 is active.

• Event-response bounds use the calibrated preheat/cutback/recovery metrics when the corresponding thermflex constraint switches are enabled.

• Non-residential hot water is intentionally zero in the current Vienna V1 building-stock path.

• Residential archetypes represent apartment-block-like multi-family reference buildings; non-residential archetypes are pragmatic service/office-like V1 proxies.

• The base residential U-values are TABULA-informed cohort seed values, not direct one-to-one TABULA WebTool extractions.

Table A. Cohort scaling and stock anchors

These values define the Vienna cohort members that enter the paper runs. They control cohort scale, annual heat anchors, hot-water inclusion, and the mapping from each cohort to a thermal archetype.

| Cohort | Sector | Period | GFA [m2] | Volume [m3] | Annual space heat [kWh/a] | Annual hot water [kWh/a] | HW? | DH share override | Load mix | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | residential | pre1975 | 48,559,035 | 145,677,642 | 5,584,485,867 | 752,665,042 | yes | n/a | H0=1.00 | BS-1, BS-2, BS-R1 |
| residential_1975_1990 | residential | 1975_1990 | 21,581,793 | 64,745,619 | 2,481,993,719 | 334,517,797 | yes | n/a | H0=1.00 | BS-1, BS-2, BS-R1 |
| residential_1990_2000 | residential | 1990_2000 | 3,596,966 | 10,790,936 | 413,665,620 | 55,752,966 | yes | n/a | H0=1.00 | BS-1, BS-2, BS-R1 |
| residential_2000_2014 | residential | 2000_2014 | 15,287,104 | 45,861,480 | 1,758,078,884 | 236,950,106 | yes | n/a | H0=1.00 | BS-1, BS-2, BS-R1 |
| non_residential_pre1975 | non_residential | pre1975 | 24,835,441 | 74,505,691 | 3,227,863,636 | 0 | no | n/a | G0=0.25; G1=0.25; G2=0.25; G3=0.25 | BS-1, BS-2, BS-R1 |
| non_residential_1975_1990 | non_residential | 1975_1990 | 11,037,974 | 33,113,640 | 1,434,606,061 | 0 | no | n/a | G0=0.25; G1=0.25; G2=0.25; G3=0.25 | BS-1, BS-2, BS-R1 |
| non_residential_1990_2000 | non_residential | 1990_2000 | 1,839,662 | 5,518,940 | 239,101,010 | 0 | no | n/a | G0=0.25; G1=0.25; G2=0.25; G3=0.25 | BS-1, BS-2, BS-R1 |
| non_residential_2000_2014 | non_residential | 2000_2014 | 7,818,565 | 23,455,495 | 1,016,179,293 | 0 | no | n/a | G0=0.25; G1=0.25; G2=0.25; G3=0.25 | BS-1, BS-2, BS-R1 |

Table B. Base envelope and geometry archetype parameters

These are the base Vienna archetype parameters before the calibrated_v1 sidecar is applied. The fields remain part of the active runtime path. The residential period ladder is TABULA-informed, but the values are simplified cohort seed values rather than direct one-to-one TABULA WebTool extractions.

| Archetype | U_wall | U_window | U_roof | U_floor | Wall/GFA | Window/GFA | Roof/GFA | Floor/GFA | Cond. floor share | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | 1.40 | 2.30 | 1.70 | 1.20 | 0.82 | 0.18 | 0.37 | 0.36 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| residential_1975_1990 | 1.10 | 2.70 | 0.80 | 0.80 | 0.82 | 0.18 | 0.37 | 0.36 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| residential_1990_2000 | 0.60 | 2.50 | 0.50 | 0.50 | 0.82 | 0.18 | 0.37 | 0.36 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| residential_2000_2014 | 0.35 | 1.40 | 0.20 | 0.40 | 0.82 | 0.18 | 0.37 | 0.36 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| non_residential_pre1975 | 1.40 | 2.30 | 1.70 | 1.20 | 0.30 | 0.12 | 0.30 | 0.30 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| non_residential_1975_1990 | 1.10 | 2.70 | 0.80 | 0.80 | 0.30 | 0.12 | 0.30 | 0.30 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| non_residential_1990_2000 | 0.60 | 2.50 | 0.50 | 0.50 | 0.30 | 0.12 | 0.30 | 0.30 | 1.00 | AR-1, AR-2, AR-5, AR-7 |
| non_residential_2000_2014 | 0.35 | 1.40 | 0.20 | 0.40 | 0.30 | 0.12 | 0.30 | 0.30 | 1.00 | AR-1, AR-2, AR-5, AR-7 |

Table C. Base thermal-mass, comfort, and window metadata

These fields are still attached to the archetype layer. Source quality is mixed: some fields are literature-backed, while others remain pragmatic V1 assumptions and are marked accordingly.

| Archetype | c_th [Wh/m2K] | T_min [C] | T_max [C] | Window typology class | Glazing source tag | Solar/shading assumption | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | 80.0 | 21.00 | 27.00 | single_glazing_box_type_or_wood_frame | AT_TABULA_ScientificReport_AEA_period_window_typologies | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| residential_1975_1990 | 75.0 | 21.00 | 27.00 | double_glazing_composite_window | AT_TABULA_ScientificReport_AEA_period_window_typologies | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| residential_1990_2000 | 70.0 | 21.00 | 27.00 | heat_protection_glazing | AT_TABULA_ScientificReport_AEA_period_window_typologies | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| residential_2000_2014 | 65.0 | 21.00 | 27.00 | triple_glazing_or_high_performance_window | AT_TABULA_ScientificReport_AEA_period_window_typologies | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| non_residential_pre1975 | 70.0 | 21.00 | 27.00 | n/a | non_residential_v1_placeholder_no_source_backed_window_typology | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| non_residential_1975_1990 | 65.0 | 21.00 | 27.00 | n/a | non_residential_v1_placeholder_no_source_backed_window_typology | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| non_residential_1990_2000 | 60.0 | 21.00 | 27.00 | n/a | non_residential_v1_placeholder_no_source_backed_window_typology | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |
| non_residential_2000_2014 | 55.0 | 21.00 | 27.00 | n/a | non_residential_v1_placeholder_no_source_backed_window_typology | TABULA_common_procedure_standard_shading_values_pending_cohort_specific_refinement | AR-2, AR-3, AR-4, AR-6, AR-R1 |

Table D. Reference-building runtime geometry and legacy solar compatibility fields

These values describe one reference building per archetype on the current teacher-scale reference body. They are not Vienna-wide sums. The reference gross floor area is taken from the building-calibration SSOT. The solar multipliers and glazing-g values are still legacy compatibility fields in the runtime path and are not yet fully cohort-specific literature-backed SSOT.

| Archetype | Ref. GFA [m2] | Ref. volume [m3] | A_floor [m2] | A_wall [m2] | A_window_total [m2] | A_roof [m2] | Room height [m] | cp_air | g_glazing | g_glazing_shaded | Solar multipliers | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | 1,000 | 3,000 | 1,000 | 820 | 180 | 370 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| residential_1975_1990 | 1,000 | 3,000 | 1,000 | 820 | 180 | 370 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| residential_1990_2000 | 1,000 | 3,000 | 1,000 | 820 | 180 | 370 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| residential_2000_2014 | 1,000 | 3,000 | 1,000 | 820 | 180 | 370 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| non_residential_pre1975 | 1,000 | 3,000 | 1,000 | 300 | 120 | 300 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| non_residential_1975_1990 | 1,000 | 3,000 | 1,000 | 300 | 120 | 300 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| non_residential_1990_2000 | 1,000 | 3,000 | 1,000 | 300 | 120 | 300 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |
| non_residential_2000_2014 | 1,000 | 3,000 | 1,000 | 300 | 120 | 300 | 3.00 | 0.34 | 0.60 | 0.00 | south=1.00; east=0.20; west=0.20; north=0.00 | RT-1, CAL-3, LEG-1 |

Table E. Calibrated reduced-order runtime parameters for the reference building

These are the EnergyPlus-teacher-derived reduced-order parameters that the current calibrated_v1 runtime path actually uses for thermal dynamics. Both the normalized per-square-meter values and the resulting parameters for the reference building are shown.

| Archetype | H_total [W/m2K] | H_trans [W/m2K] | C_eff [Wh/m2K] | tau [h] | H_total ref [W/K] | H_trans ref [W/K] | C_eff ref [Wh/K] | Air-loss scale | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | 1.264 | 1.264 | 25.020 | 19.79 | 1,264.4 | 1,264.4 | 25,019.7 | 0.000 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_1975_1990 | 1.045 | 1.038 | 22.253 | 21.30 | 1,044.8 | 1,038.1 | 22,253.4 | 0.014 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_1990_2000 | 0.791 | 0.734 | 20.432 | 25.83 | 791.2 | 734.1 | 20,432.0 | 0.115 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_2000_2014 | 0.478 | 0.342 | 21.196 | 44.31 | 478.4 | 341.7 | 21,195.8 | 0.276 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_pre1975 | 0.900 | 0.900 | 25.705 | 28.55 | 900.4 | 900.4 | 25,704.5 | 0.000 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_1975_1990 | 0.719 | 0.673 | 22.112 | 30.77 | 718.7 | 673.5 | 22,112.0 | 0.091 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_1990_2000 | 0.544 | 0.480 | 19.504 | 35.84 | 544.3 | 479.5 | 19,504.3 | 0.131 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_2000_2014 | 0.322 | 0.208 | 19.192 | 59.64 | 321.8 | 208.2 | 19,191.7 | 0.229 | CAL-1, CAL-2, CAL-3, CAL-4 |

Table F. Calibrated event-response parameters

These are the cohort-specific thermflex sidecar parameters fitted from the EnergyPlus teacher experiments. They constrain preheat, cutback, and recovery behavior when event-response bounds are active.

| Cohort | Preheat energy [kWh] | Preheat peak [kW] | Cutback shed [kWh] | Cutback peak [kW] | Recovery rebound [kWh] | Recovery peak [kW] | Recovery time [h] | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| residential_pre1975 | 79.01 | 12.31 | 76.11 | 11.54 | 50.24 | 7.93 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_1975_1990 | 71.96 | 11.71 | 69.69 | 10.82 | 50.75 | 7.96 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_1990_2000 | 64.76 | 10.86 | 60.40 | 9.83 | 49.29 | 7.65 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| residential_2000_2014 | 51.12 | 9.07 | 44.70 | 7.08 | 40.60 | 5.89 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_pre1975 | 67.66 | 11.19 | 64.42 | 10.23 | 49.20 | 7.78 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_1975_1990 | 61.38 | 10.53 | 58.42 | 9.39 | 48.49 | 7.63 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_1990_2000 | 55.24 | 9.67 | 50.88 | 8.21 | 45.23 | 7.02 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |
| non_residential_2000_2014 | 42.99 | 7.48 | 33.75 | 4.05 | 30.73 | 4.50 | 1.00 | CAL-1, CAL-2, CAL-3, CAL-4 |

Table G. Global heating-control parameters

These are global building-operation settings from the Settings layer. They are not archetype-specific and may be overridden by explicit paper-case JSON overrides, but they still belong to the active building model path.

| Parameter | Value | Role | Sources |
| --- | --- | --- | --- |
| reference_control_mode | constant | global default | HC-1 |
| control_mode | constant | global default | HC-1 |
| constant_setpoint_c | 22.00 | global default | HC-1 |
| day_setpoint_c | 22.00 | global default | HC-1 |
| night_setpoint_c | 19.00 | global default | HC-1 |
| day_start_hour | 6 | global default | HC-1 |
| night_start_hour | 22 | global default | HC-1 |
| hysteresis_band_k | 1.00 | global default | HC-1 |
| comfort_band_enabled | no | global default | HC-1 |
| comfort_band_k | 0.00 | global default | HC-1 |
| max_heating_power_mode | archetype_design | global default | HC-1 |
| max_heating_power_w_per_m2 | 60.0 | global default | HC-1 |
| max_heating_power_multiplier | 1.00 | global default | HC-1 |
| design_indoor_temp_c | 22.00 | global default | HC-1 |
| design_outdoor_temp_c | -12.00 | global default | HC-1 |
| design_ventilation_mode | p95 | global default | HC-1 |
| design_internal_gains_w_per_m2 | 0.00 | global default | HC-1 |
| design_solar_gains_w_per_m2 | 0.00 | global default | HC-1 |
| enable_active_cooling | yes | global default | HC-1 |
| cooling_setpoint_c | 27.00 | global default | HC-1 |
| max_cooling_power_w_per_m2 | 40.0 | global default | HC-1 |

Table H. Global thermflex constraint parameters

These are the global thermflex/bound settings from the Settings layer. Representative-day and paper sensitivity runs override a subset of them.

| Parameter | Value | Role | Sources |
| --- | --- | --- | --- |
| use_explicit_lower_bounds | no | scenario-dependent | TF-1 |
| constant_lower_bound_c | n/a | scenario-dependent | TF-1 |
| day_lower_bound_c | n/a | scenario-dependent | TF-1 |
| night_lower_bound_c | n/a | scenario-dependent | TF-1 |
| comfort_band_k | 0.00 | scenario-dependent | TF-1 |
| reference_deadband_k | 0.50 | scenario-dependent | TF-1 |
| constrain_upper_temperature | no | scenario-dependent | TF-1 |
| use_event_response_bounds | no | scenario-dependent | TF-1 |
| enforce_event_peak_bounds | yes | scenario-dependent | TF-1 |
| enforce_event_energy_bounds | yes | scenario-dependent | TF-1 |
| enforce_recovery_cooldown | yes | scenario-dependent | TF-1 |
| max_flex_duration_h | 0 | scenario-dependent | TF-1 |
| max_flex_events_per_day | 0 | scenario-dependent | TF-1 |
| activation_penalty_eur_per_member_h | 0.000100 | scenario-dependent | TF-1 |
| temperature_violation_penalty_eur_per_degree_h | 1,000,000 | scenario-dependent | TF-1 |
| allow_terminal_deviation | yes | scenario-dependent | TF-1 |
| terminal_band_k | 0.00 | scenario-dependent | TF-1 |

Bibliography and Source Notes

BS-1: Citiwatt indicators Vienna local snapshot; used via Data/building_stock/Vienna/building_stock.py.

Note: Building-stock scale anchors: total heat, GFA, volume, and period shares.

BS-2: Vienna Energy Report 2025 and repo notes in Documentation/Sources/wien_und_dispatch_quellen.md.

Note: Official Vienna electricity anchors and building-sector context used in the stock layer.

BS-R1: Repo-derived V1 stock assumptions in Data/building_stock/Vienna/building_stock.py.

Note: Includes the explicit non-residential hot-water exclusion and the current exogenous electricity calibration anchors.

AR-1: Austrian TABULA / EPISCOPE country page: https://episcope.eu/building-typology/country/at/

Note: Residential period-specific U-value and typology anchor.

AR-2: Austrian TABULA Scientific Report: https://episcope.eu/fileadmin/tabula/public/docs/scientific/AT_TABULA_ScientificReport_AEA.pdf

Note: Residential period ladder, window typologies, and Austrian multi-family context.

AR-3: TABULA common calculation procedure: https://episcope.eu/building-typology/tabula-structure/calculation/

Note: Standard shading and usage procedure values; method anchor, not cohort-specific Vienna truth.

AR-4: TABULA WebTool FAQ: https://episcope.eu/building-typology/webtool/

Note: Important caution that the 45 Wh/(m2K) procedure value is too coarse for research use.

AR-5: TABULA Final Report Appendix Volume: https://episcope.eu/fileadmin/tabula/public/docs/report/TABULA_FinalReport_AppendixVolume.pdf

Note: Residential envelope-area ratios per conditioned floor area.

AR-6: OIB-Richtlinie 6 / Kostenoptimalitaet: https://www.oib.or.at/sites/default/files/kostenoptimalitaet_0.pdf

Note: Austrian plausibility anchors for setpoints, g-values, and air-change assumptions.

AR-7: TABULA Thematic Report No. 3 / Non-Residential Buildings: https://episcope.eu/building-typology/tabula-structure/non-residential/

Note: Explains why non-residential geometry and exposure assumptions remain less certain than residential ones.

AR-R1: Repo-internal V1 assumption in Data/thermal_archetypes/Vienna/thermal_archetypes.py.

Note: Areal heat capacities and some non-residential fields are still pragmatic start values, not direct Vienna observations.

RT-1: Runtime derivation in Technical_model/technologies/buildings/runtime_building_params.py.

Note: Cohort-scale areas, room height, and effective runtime fields are derived from the stock + archetype SSOT.

LEG-1: Legacy compatibility fields in Data/technology_data/building.py and runtime_building_params.py.

Note: solar_multipliers, g_glazing, and g_glazing_shaded remain active compatibility placeholders and are not yet fully recalibrated Vienna SSOT.

CAL-1: Open-Meteo Historical Weather API: https://open-meteo.com/en/docs/historical-weather-api

Note: Weather driver for the pseudo-EPW and calibration path.

CAL-2: Climate.OneBuilding Vienna EPW template: https://climate.onebuilding.org/WMO_Region_6_Europe/AUT_Austria/index.html

Note: Header and format anchor for pseudo-EPW generation.

CAL-3: EnergyPlus 26.1.0 and repo calibration teacher path.

Note: Teacher model used to derive reduced-order and event-response fits.

CAL-4: Repo-generated calibration exports: Data/thermal_archetypes/Vienna/calibrated_v1.py plus reduced_order_fit_summary.csv and event_response_fit_summary.csv.

Note: Direct source of the fitted reduced-order and event-response sidecar parameters.

HC-1: Settings/technical/heating_control.py

Note: Global heating and cooling control defaults; scenario-specific overrides may change them in paper runs.

TF-1: Settings/constraints/thermflex.py

Note: Global thermflex-bound defaults; representative-day and paper runs override a subset of them explicitly.